# Advanced IoT & AI Development Kit

Core System and components : Raspberry Pi 5 (8GB) & ESP32-S3 Ecosystem Focus: Edge AI, Computer Vision, and Distributed IoT Protocols

## 1. Description & Component Overview

This laboratory kit is a high-performance development environment designed for Artificial Intelligence (AI) and Internet of Things (IoT) applications. The Raspberry Pi 5 serves as the central processing hub for Computer Vision (Open-CV), while the Fire-Beetle 2 ESP32-S3 and ESP32-S3-CAM act as edge nodes for real-time sensing and remote image capture.

### Hardware Component Gallery

| Component | Function |
| --- | --- |
| Raspberry Pi 5 (8GB) | Main computer for AI, Open-CV, and programming. |
| Fire-Beetle 2 ESP32-S3 | IoT hub for real-time control and WiFi/BLE. |
| ESP32-S3-CAM | Distributed image capture and video streaming. |
| 27W USB-C Power Supply | Required for stable operation of Pi 5 (5.1V/5A). |
| 32GB Micro SD Card | Stores Raspberry Pi OS and project files. |
| Micro HDMI to HDMI | Connects the Pi 5 to a standard monitor. |
| Active Cooler & Case | Protects the board and manages high thermals. |
| Connector Pins/Headers | For soldering/interfacing the Fire-Beetle 2. |

## 2. Technical Pin-out & Functional Descriptions

### A. Raspberry Pi 5 (40-Pin Header)

- Logic Level: 3.3V (Strict). Do not apply 5V to GPIO pins.

- Max Current: ~16mA per pin.

| Pin No. | Name | Type | Description |
| --- | --- | --- | --- |
| 1, 17 | 3.3V | Power | DC power output for low-power sensors. |
| 2, 4 | 5V | Power | DC power output for fans or 5V peripherals. |
| 3, 5 | SDA / SCL | I2C | Communication for OLEDs and digital sensors. |
| 6, 9, 14... | GND | Ground | Common reference for the circuit. |
| 8, 10 | TXD / RXD | UART | Serial communication/debugging interface. |
| 12, 32, 33 | PWM | Output | Controls motor speed or LED dimming. |
| 19, 21, 23 | MOSI/MISO/SCLK | SPI | High-speed data for displays or RFID readers. |

### B. Fire-Beetle 2 ESP32-S3 Pin-out

- Logic Level: 3.3V.

- Specialty: Optimized for low-power battery operation.

| Label | Function | Description |
| --- | --- | --- |
| VCC | Power Input | USB (5V) or Li-ion Battery (3.7V) input. |
| 3V3 | Power Output | Regulated 3.3V output for external modules. |
| A0 - A4 | ADC | Analog-to-Digital: Reads variable voltage sensors. |
| GPIO 1-21 | Digital I/O | Standard pins for buttons, LEDs, and relays. |
| SDA / SCL | I2C | Connects to the Pi 5 or I2C sensors. |
| BOOT/RST | Control | Used for firmware flashing and system resets. |

## 3. How to Operate (Step-by-Step)

### Phase 1: Flashing the Raspberry Pi OS

- Install Tool: Download Raspberry Pi Imager on your PC.

- Select Device: Choose Raspberry Pi 5.

- Select OS: Choose Raspberry Pi OS (64-bit).

- Insert SD: Plug your 32GB SD card into the PC.

- Write: Click Next. Use "Edit Settings" to pre-configure Wi-Fi and SSH. Click Write.

- Boot: Insert the card into the Pi 5, connect the 27W Power Supply, and connect to a monitor.

### Phase 2: Programming the ESP32-S3

- Environment: Open Arduino IDE.

- Board Manager: Go to File > Preferences. Add: https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Install Core: In Tools > Board > Boards Manager, search for ESP32 and click Install.

- Select Board: Set board to DFRobot FireBeetle 2 ESP32-S3.

- Connect: Use a USB-C Data Cable. Select the correct Port.

- Upload: Write your code and click Upload. (Hold BOOT if the upload fails).

## 4. Do’s and Don’ts

| Do | Don't |
| --- | --- |
| Do keep the Active Cooler running; Pi 5 throttles at 80°C. | Don't hot-swap the SD card while the Green LED is flickering. |
| Do use the official 27W supply to ensure 5A current delivery. | Don't connect 5V sensors to the 3.3V GPIO pins. |
| Do use a common GND when wiring the Pi and ESP32 together. | Don't leave the ESP32-S3-CAM in direct sunlight for long periods. |

## 5. Safety Measures

- Thermal Safety: The Pi 5 processor is an exposed die. Avoid touching it during operation to prevent burns.

- Electrical Safety: Always disconnect the USB-C Power before changing any wiring on the GPIO headers.

- Shutdown Protocol: Always use sudo halt or the GUI shutdown to prevent SD card corruption.

- Static Protection: Touch a grounded metal object before handling the Fire-Beetle 2 to discharge static electricity.

Components Images:

1.RaspBerry pi 5

2. Fire-bettle esp 32

3.esp 32 cammera module

4.RaspBerry pi case

5. 27 W power Adapter for RaspBerry pi

6.Micro HDMI to HDMI Cable

7.32 GB SD Card

Links for raspberry pi configuration

https://youtu.be/ZH6vfvRstfM?si=Vp0DY1wYyE-MQz4J

link for firebettle esp 32

https://youtu.be/4g1uRzDVb6U?si=RbMwezJBjF2F-d3j